# Vehicle Model Parameters

Epic Quest 1 | Saved model snapshot: September 27, 2026

The Jetta and Camry models average the fitted coefficients of three trials. Each table is sized for one IEEE report column. Values are rounded to six significant digits; calculations retain full precision.

# Mean trial motion models

τ is local phase time (s). Acceleration is fitted linearly; velocity and position are its integrals, not separate regressions.

a(τ) = a₁τ + a₀

v(τ) = v₂τ² + v₁τ + v₀

x(τ) = x₃τ³ + x₂τ² + x₁τ + x₀

TABLE I. MEAN TRIAL ACCELERATION COEFFICIENTS

| Term | Unit | Jetta | Camry |
| --- | --- | --- | --- |
| a₁ | ft/s³ | 1.62908 | 0.239433 |
| a₀ | ft/s² | 1.99253 | 1.96674 |
| v₂ | ft/s³ | 0.81454 | 0.119716 |
| v₁ | ft/s² | 1.99253 | 1.96674 |
| v₀ | ft/s | 1.51891 | 2.73403 |
| x₃ | ft/s³ | 0.271513 | 0.0399054 |
| x₂ | ft/s² | 0.996266 | 0.983369 |
| x₁ | ft/s | 1.51891 | 2.73403 |
| x₀ | ft | 0.101886 | 0.0742746 |

TABLE II. MEAN TRIAL BRAKING COEFFICIENTS

| Term | Unit | Jetta | Camry |
| --- | --- | --- | --- |
| a₁ | ft/s³ | -1.81889 | -0.204728 |
| a₀ | ft/s² | -3.04233 | -3.82152 |
| v₂ | ft/s³ | -0.909445 | -0.102364 |
| v₁ | ft/s² | -3.04233 | -3.82152 |
| v₀ | ft/s | 53.1618 | 51.0353 |
| x₃ | ft/s³ | -0.303148 | -0.0341213 |
| x₂ | ft/s² | -1.52117 | -1.91076 |
| x₁ | ft/s | 53.1618 | 51.0353 |
| x₀ | ft | 182.273 | 348.851 |

Tables I–II retain the recorded phase starting speed and position. Course prediction instead reintegrates a(τ) with the starting conditions in Tables XI–XII. Do not substitute these mean trial v₀ and x₀ values into the course equations.

TABLE III. SAVED IDLE MOTION CONSTANTS

| Vehicle / phase | a (ft/s²) | v (ft/s) | x (ft) |
| --- | --- | --- | --- |
| Jetta / initial | 0 | 0 | 0 |
| Jetta / final | 0 | 0 | 349.451 |
| Camry / initial | 0 | 0 | 0 |

The saved Camry model has no fitted final-idle phase.

# Engine RPM and fuel models

Here u is vehicle speed in mph, n is engine RPM, and f is fuel rate in US gal/s. These slopes must not be evaluated using speed in ft/s.

n(u) = mᴿu + bᴿ;    f(u) = mᶠu + bᶠ

TABLE IV. ACCELERATION: RPM AND FUEL

| Term | Unit | Jetta | Camry |
| --- | --- | --- | --- |
| mᴿ | rpm/mph | 55.9871 | 23.7418 |
| bᴿ | rpm | 1990.95 | 1625.72 |
| mᶠ | (gal/s)/mph | 3.981e-05 | 1.36733e-05 |
| bᶠ | gal/s | 0.000653753 | 0.000328541 |

TABLE V. BRAKING: RPM AND FUEL

| Term | Unit | Jetta | Camry |
| --- | --- | --- | --- |
| mᴿ | rpm/mph | 62.455 | 21.3401 |
| bᴿ | rpm | 466.347 | 595.354 |
| mᶠ | (gal/s)/mph | -1.83079e-11 | -2.6823e-06 |
| bᶠ | gal/s | 0.000143361 | 0.000108384 |

TABLE VI. IDLE RPM AND OBSERVED SPEED LIMIT

| Parameter | Jetta | Camry |
| --- | --- | --- |
| Idle RPM (rpm) | 928.208 | 685.418 |
| Max measured speed (mph) | 37.2823 | 35.4182 |

Prediction limits engine RPM below by the idle value and fuel rate below by zero. Cruise values are midpoint estimates of the acceleration and braking fits at cruise speed (Table XIII), not separate measured cruise regressions.

RPM fitting requires positive speed and RPM; fuel fitting requires nonnegative fuel rate only. Before averaging, a trial fuel line negative at 0 or 25 mph is replaced by its phase mean, making that phase fuel model constant with speed.

TABLE VII. RECORDED PHASE DURATIONS

| Vehicle / phase | Mean (s) | Max (s) | Trials |
| --- | --- | --- | --- |
| J: Initial idle | 3.448 | 7.122 | 3 |
| J: Accel | 5.37067 | 5.93 | 3 |
| J: Brake | 7.36533 | 10.203 | 3 |
| J: Final idle | 1.08233 | 1.508 | 3 |
| C: Initial idle | 0.112667 | 0.152 | 3 |
| C: Accel | 14.2433 | 16.612 | 3 |
| C: Brake | 9.73033 | 10.851 | 3 |
| C: Final idle | — | — | — |

J = Jetta; C = Camry; — = no saved fit. Recorded durations are not the predicted course durations. Idle RPM values follow the saved selections: Jetta initial-idle indices; Camry RPM < 900, including zero readings.

Source: ModelPredictionData.mat (VehicleModels, Courses, Predictions). Motion/RPM/fuel fits originate from DataParser.mat; idle summaries are loaded from JettaData.mat and CamryData.mat by ModelPrediction.m.

# Course definitions and shared settings

TABLE VIII. COURSE INPUTS

| Course | Distance (ft) | Condition |
| --- | --- | --- |
| 1 | 300 | Reach 15 mph; cruise |
| 2 | 600 | Reach 25 mph; stop |
| 3 | 1320 | Average 15 mph |
| 4 | 1320 | Accelerate throughout |

Course 3 has 60 s of travel time. Its solved cruise speed is above 15 mph because the acceleration phase starts from rest.

TABLE IX. CURRENT SOURCE-CODE SETTINGS

| Setting | Value |
| --- | --- |
| Maximum prediction step | 0.05 s |
| Idle RPM plot span | 15 s |
| Camry gasoline density | 6.17 lb/US gal |
| mph to ft/s multiplier | 5280/3600 |
| Root imaginary tolerance | 10⁻⁹ s |
| Cruise distance rounding | 10⁻⁸ ft |
| Idle speed threshold | 0.2 ft/s |
| Adjacent speed-change threshold | 0.2 ft/s |

Actual prediction spacing may be smaller than 0.05 s. The motion detector uses adjacent speed differences, not acceleration.

All courses display 15 s of initial idle RPM; Course 2 also displays final idle. Idle displays are excluded from travel time and fuel totals. The current prediction has no idle fuel-rate coefficient.

# Kalman filter settings

TABLE X. FILTER INITIALIZATION

| Parameter | Value / unit |
| --- | --- |
| State order | [speed; acceleration] |
| Initial state | [0; 0] |
| Initial covariance P₀ | 2 × 2 zero matrix |
| Measurement matrix H | [1 0] |
| Measurement variance R | 1 (ft/s)² |
| Initial process intensity q₀ | 1 ft²/s⁵ |

F = [1, Δt; 0, 1]

Q = q [Δt³/3, Δt²/2; Δt²/2, Δt]

Semicolons separate matrix rows. Δt comes from recorded timestamps. Q is rebuilt each sample; q is updated for the next step using this adaptive tuning rule:

var(gradient(a,t).*dt)/dt

a and t are the acceleration history and matching timestamps; dt is the current interval. This rule estimates q, rather than measuring the true noise covariance.

Tables IX–X: current settings in ModelPrediction.m, SpeedKalmanFilter.m, and detectMotionSections.m; not saved MAT fields.

# Course-specific initial conditions

Use local τ = t − tstart and the appropriate acceleration coefficients from Tables I–II. Cruise uses a₁ = a₀ = 0. These expressions give every course-specific coefficient without repeating the common terms:

a(τ) = a₁τ + a₀

v(τ) = (a₁/2)τ² + a₀τ + v₀

x(τ) = (a₁/6)τ³ + (a₀/2)τ² + v₀τ + x₀

TABLE XI. JETTA PHASE CONDITIONS

| Course / phase | tstart (s) | Duration (s) | v₀ (ft/s) | x₀ (ft) |
| --- | --- | --- | --- | --- |
| 1/A | 0.000 | 4.116 | 0.000 | 0.000 |
| 1/C | 4.116 | 12.009 | 22.000 | 35.809 |
| 2/A | 0.000 | 5.597 | 0.000 | 0.000 |
| 2/C | 5.597 | 11.283 | 36.667 | 78.808 |
| 2/B | 16.880 | 4.894 | 36.667 | 492.521 |
| 3/A | 0.000 | 4.227 | 0.000 | 0.000 |
| 3/C | 4.227 | 55.773 | 22.981 | 38.318 |
| 4/A | 0.000 | 15.802 | 0.000 | 0.000 |

TABLE XII. CAMRY PHASE CONDITIONS

| Course / phase | tstart (s) | Duration (s) | v₀ (ft/s) | x₀ (ft) |
| --- | --- | --- | --- | --- |
| 1/A | 0.000 | 7.636 | 0.000 | 0.000 |
| 1/C | 7.636 | 10.222 | 22.000 | 75.115 |
| 2/A | 0.000 | 11.119 | 0.000 | 0.000 |
| 2/C | 11.119 | 7.363 | 36.667 | 176.415 |
| 2/B | 18.482 | 7.916 | 36.667 | 446.406 |
| 3/A | 0.000 | 8.099 | 0.000 | 0.000 |
| 3/C | 8.099 | 51.901 | 23.782 | 85.706 |
| 4/A | 0.000 | 25.647 | 0.000 | 0.000 |

A = acceleration; C = cruise; B = braking. Times and starting conditions above are rounded to three decimals for readability. Full-precision coefficient vectors remain in Predictions; PrintModelParameters.m prints each vector.

TABLE XIII. PREDICTED CRUISE CONSTANTS

| Vehicle / course | Engine RPM (rpm) | Fuel rate (US gal/s) |
| --- | --- | --- |
| Jetta / 1 | 2116.963 | 0.000697132 |
| Camry / 1 | 1448.649 | 0.000300895 |
| Jetta / 2 | 2709.174 | 0.000896182 |
| Camry / 2 | 1674.059 | 0.00035585 |
| Jetta / 3 | 2156.555 | 0.000710439 |
| Camry / 3 | 1476.033 | 0.000307571 |

Course 4 has no cruise or braking phase. Predictions are flagged as extrapolated when acceleration duration exceeds the longest measured acceleration phase or reached speed exceeds the measured maximum. Both Course 4 predictions are flagged; they are not independently validated high-speed performance.